Experiment 1:- Analyze the Computer Network Design using SWITCH and HUB in GNS3.

Switch Configure:

Fig: Switch

- Drag &Drop 4 VPCs and make the required connections and click on start option to run all devices.

- Set ip address for all VPCs as follows:

PC1>192.168.1.1/24 192.168.1.254

PC2>192.168.1.2/24 192.168.1.254

PC3>192.168.1.3/24 192.168.1.254

PC4>192.168.1.4/24 192.168.1.254

Fig: IP address for all 4 PC’s.

- Right click on PC4 & give start capture.

Fig: PC4 Start Capture Picture

4. PC1>ping 192.168.1.2         ->no data transfer

5. PC1>ping 192.168.1.3         ->no data transfer

6. PC1>ping 192.168.1.4         ->data transfer

Fig: Wireshark Displaying the request reply through the connected VPCs and Captured PC.

HUB Configuration:-

Fig: Hub

- Drag & Drop 4 VPCs

- Add Ethernet Hub and make a connection..

- Set IP Address for all the VPCs.

Fig: setting IP address for all VPCs.

- Right click on PC4 and give Start Capture.

Fig: - PC4 Start Capture Picture

- Ping all the VPCs  using ping command

- PC1>ping 192.168.1.2

- PC1>ping 192.168.1.3

- PC1>ping 192.168.1.4

Output:-

Fig: - Wireshark is displaying the request reply through the connected VPCs and Captured PC.

Experiment 2: - Analyze the Router in Computer Network Design using GNS3.

Objectives:

- To Learn about IP address Assignment for different sub networks

- To study the functions of ROUTER device

- To study the functions of SWITCH device

To start with the Lab exercise, create a topology as shown in Figure below.

Single router:-

Fig:- Single Router Configuration

- Drag & Drop 4 VPCs, 2 Switches, 1 Router

- Make all the required connections and click on start button to start all the devices.

- Set IP Address for all the VPCs as follows:

PC1>192.168.1.1/24 192.168.1.254

PC2>192.168.1.2/24 192.168.1.254

PC3>192.168.2.1/24 192.168.2.254

PC4>192.168.2.2/24 192.168.2.254

Fig:- Showing PC’s IP address.

- Click on PC1 and start capture

- PC1>ping 192.168.1.1    ->data transfer takes place

- PC1>ping 192.168.2.1    ->Host not reachable

Fig: Ping PC1 to PC3.

Router Configuration:-

Router Configuration Commands:

R1# config t

R1 (config)#int f0/0

R1 (config)#ip address 192.168.1.254 255.255.255.0

R1 (config)#no shut

R1 (config)#exit

R1 (config)#int f1/0

R1 (config)# ip address 192.168.2.254 255.255.255.0

R1 (config)#no shut

R1 (config)#exit

- Ping PC1 to PC3.

PC1> ping 192.168.2.1

Fig:  Data Packets transfer from PC1 to PC3.

Experiment 3:- Analyze the Computer Network Design using Two Routers in GNS3.

Double Router Configuration:

Fig: - Double Router Configuration

- Drag & Drop 4 VPCs and 2 Switch and also add 2 Routers and make connection is shown in above figure.

- Click on start button to start all devices.

- Set Ip Address for all the VPCs as follows:

PC1>192.168.1.1/24 192.168.1.254

PC2>192.168.1.2/24 192.168.1.254

PC3>192.168.2.1/24 192.168.2.254

PC4>192.168.2.2/24 192.168.2.254

Fig:- Showing PC’s IP address.

- Click on PC1 and start capture

- PC1>ping 192.168.1.1    ->data transfer takes place

- PC1>ping 192.168.2.1    ->Host not reachable

- Click on Router R1 and Write the below commands

R1#config t

R1 (config)#int f0/0

R1 (config)#ip address 192.168.1.254 255.255.255.0

R1 (config)#no shut

R1 (config)#exit

R1 (config)#int f1/0

R1 (config)#ip address 192.168.5.1 255.255.255.0

R1 (config)#no shut

R1 (config)#exit

- Click on Router R2 and write below commands.

R2#config t

R2 (config)#int f0/0

R2 (config)#ip address 192.168.2.254 255.255.255.0

R2 (config)#no shut

R2 (config)#exit

R2 (config)#int f1/0

R2 (config)#ip address 192.168.5.2 255.255.255.0

R2 (config)#no shut

R2 (config)#exit

- After the configuration, PC1>ping 192.168.2.1

PC1>ping 192.168.1.2

- Now give the Route for transferring the data through the 2 switches using below commands.

For Router R1:

R1#ip route 192.168.2.0 255.255.255.0 192.168.5.2

For Router R2:

R2#ip route 192.168.1.0 255.255.255.0 192.168.5.1

Now ping the VPCs.

PC1>ping 192.168.2.1

Experiment 4: -Analyze the Dynamic Host Configuration Protocol (DHCP) using GNS3.

Objectives: -

- Understand DHCP Service

- Analyzing DHCP Packets

- Understanding significance of Netmask value

DHCP Overview: -

The Dynamic Host Configuration Protocol (DHCP) is a network protocol that automatically assigns IP addresses to devices on a network.

Fig:- Network Topology for DHCP Configuration

- Drag and drop 2 VPCs, 1 Switch, 1 Router and make the connections as shown above.

- Start all the devices.

- In order to configure our router as a DHCP server the following commands are used.

Router Configuration:-

R1#config t

R1(config)#int f0/0

R1(config)#ip address 192.168.1.254 255.255.255.0

R1(config)#no shut

R1(config)#exit

R1(config)#ip dhcp pool formation

R1(dhcp-config) #network 192.168.1.0 255.255.255.0

R1(dhcp-config) #default-router 192.168.1.254

R1(dhcp-config) #dns-server 192.168.1.254

R1(dhcp-config) #class ec2lt

R1(config-dhcp-pool-class) #address range 192.168.1.2 192.168.1.4

R1(config-dhcp-pool-class) #exit

R1(dhcp-config) #exit

R1(dhcp-config) #do wr

Fig: - Router Configuration

4. The commands above will turn the interface on and assign an IP address.

5. Click the VPCS.

PC1>ip dhcp

Fig:-Showing PC1 IP address.

DDOORA 192.168.1.2 192.168.1.254 is the output

PC2>ip dhcp

Fig:-Showing PC2 IP address.

DDOORA 192.168.1.4 192.168.1.254 is the output

Experiment 5: - Analyze the design of VLAN’S using GNS3.

Objectives:

- To understand Virtual Lan (VLAN) Concepts

Fig: - VLANs Configuration

1.Set IP address for all VPCS as follows:

PC1>192.168.1.1/24

PC2>192.168.1.2/24

PC3>192.168.1.3/24

PC4>192.168.1.4/24

Fig:- IP address for all the PC’S.

2. After the IP address setting, ping the PC’s.

PC1> ping 192.168.1.3

Fig: Ping PC1 to PC3

3. Before setting a LAN connection for VPCs we should remove the wired connection.

Fig: - deleting all the connections through the VPCS.

- SWITCH Configuration: -

| PORT | VLAN |
| --- | --- |
| 0(PC1) | 1-> ADD |
| 1(PC2) | 1-> ADD |
| 2(PC3) | 2-> ADD |
| 3(PC4) | 2->ADD |

Check-> add-> Apply->&ok.

- PC1>ping 192.168.1.3